Will Harper

CS 499 Computer Science Capstone

Milestone Two

16 November 2025

Enhancement One: Software Design/Engineering

The artifact, Thermostat.py, is a Python program crafted to manage a functional Internet of Things (IoT) thermostat prototype that is built on a Raspberry Pi. This prototype incorporates a variety of hardware components, such as an AHT20 temperature/humidity sensor, a 16x2 character LCD, two PWM LEDs (Red for heating and Blue for cooling), along with three physical buttons, all connected through GPIO and I2C.

The code features a state machine with three distinct states: Off, Heat, and Cool. Its primary functions include reading the current temperature, allowing the user to modify a temperature setpoint, controlling the LEDs to visually represent the current state and whether the system is actively heating (with Red pulsing) or cooling (with Blue pulsing), displaying status information (date/time, current temperature, setpoint) on the LCD, and transmitting a comma-separated status string over the serial port (UART) every 30 seconds to an external TemperatureServer.

I chose the Thermostat.py Python code for a Raspberry Pi based smart thermostat from CS 350: Emerging Systems Architecture and Technologies course, as it serves as a thorough example of embedded systems programming, object-oriented design, and concurrent programming.

| Skill | Component/Code Feature |
| --- | --- |
| Object-Oriented Design(OOD) | The use of classes like ManagedDisplay and TemperatureMachine to encapsulate related functionality and data. TemperatureMachine inherits from StateMachine, demonstrating polymorphism and design patterns. |
| State Machine Implementation | Using the statemachine library to manage the three primary operational modes (off, heat, cool) and defining transitions (cycle) and actions (on_enter_heat, on_exit_cool). This is crucial for robust control logic. |
| Hardware Interfacing & I/O | Direct communication with I2C devices (adafruit_ahtx0 sensor) and managing various GPIO components (PWMLED, Button, characterlcd). This demonstrates proficiency in controlling physical hardware |
| Concurrency & Threading | The use of threading.Thread to run the manageMyDisplay method in the background (tsm.run()). This allows the main program loop to remain responsive to button presses and prevents display updates from blocking other core thermostat functions. |
| Serial Communication | Configuration and use of the serial package to implement serial (UART) output for external communication, a foundational skill in distributed IoT systems. |

The artifact was enhanced by applying defensive programming principles. Setpoint Input Validation,  I established MIN_SETPOINT (60°F) and MAX_SETPOINT (85°F) constants and incorporated bounds checking into the processTempIncButton and processTempDecButton functions. This measure prevents users from setting unreasonable or unsafe temperatures, thereby boosting the system's robustness and reliability. Then Reliable Serial Communication Logic, I addressed a potential problem in the manageMyDisplay thread where the counter variable was being reset. By ensuring that the counter only resets when the 30-second serial message is successfully transmitted, I ensure that the status update is consistently sent every 30 seconds, which enhances the accuracy of system reporting. Finally, Code Maintainability, I implemented a SERIAL_PORT constant to replace the hardcoded string /dev/ttyS0 in the serial.Serial() initialization, adhering to best practices for configuration management.

The improvement of the Thermostat.py artifact successfully aligns with the course outcome of demonstrating an ability to use well-founded and innovative techniques, skills, and tools in computing practices for the purpose of implementing computer solutions that deliver value and accomplish industry-specific goals. This was achieved by showcasing OOD principles in class design and the application of state machines, by connecting the Raspberry Pi GPIO/I2C with Python libraries for the sensor, display, and buttons, as well as by implementing threading to handle display and serial updates without hindering the main loop or button handlers.

The improvement process highlighted the essential role of input validation in embedded systems. In a tangible device such as a thermostat, an extreme setpoint might result in mechanical degradation, energy inefficiency, or safety issues. Implementing the bounds check served as a valuable lesson in ensuring that the software is both safe and operational. Additionally, I acquired a more profound insight into managing a counter variable within a thread executing a time-based loop to guarantee precise timing for scheduled events (the 30-second serial update).

The main difficulty lay in examining the timing logic within manageMyDisplay. The initial code setup reset the counter in a manner that functioned correctly only if the complete loop iteration lasted precisely one second. Yet, even a slight delay in other processes could disrupt the timing. Troubleshooting a time-sensitive problem in a concurrent thread demands meticulous consideration. By confining the counter = 1 reset to the if((counter % 30) == 0): block, I clarified the ambiguity and guaranteed that the 30-second interval is preserved accurately, irrespective of minor fluctuations in the loop.